SIA 3.2 MWp — EPC Build Options Analysis (Internal)

CONFIDENTIAL — Lighthief Cyprus Ltd — May 2026  |  Not for distribution

# 1. Overview

Four PV build configurations are compared for the SIA 3.2 MWp + 10 MWh BESS project (Larnaca District, Cyprus). All use AE Solar 645W bifacial panels (€0.112/W EXW Rotterdam, 4,961 panels) and 9 × Huawei SUN2000-330KTL-H1 inverters (Trikkis pricelist €7,329.01 pallet, March 2026). BESS: 2.5 MW / 10 MWh, 4h, Lighthief installed €125k/MWh. Revenue modelled on 50% curtailment, 87.4% BESS capture, 86.32% RTE, €195/MWh discharge (TSOC DAM Oct 2025–Feb 2026). CIT 12.5%.

# 2. Build Options

## Option A: Fixed Tilt 15° — Standard

South-facing 15° fixed tilt. Bifacial 645W. No ground treatment. Lowest CAPEX, lowest yield.

## Option B: Fixed Tilt 25° + White Albedo

Optimised 25° tilt + white albedo (crushed limestone/PVC). No permit change. RECOMMENDED.

## Option C: Single-Axis Tracker + Bifacial

SAT tracking +15% yield vs fixed. Requires permit amendment. Higher O&M.

## Option D: SAT + Bifacial + White Albedo

Maximum yield config. Permit amendment required. Highest CAPEX and complexity.

# 3. Comparison — All Four Options

| Metric | A Fixed 15° | B Fixed 25°+Albedo | C SAT Tracker | D SAT+Albedo |
| --- | --- | --- | --- | --- |
| Yield (kWh/kWp/yr) | 1,650 | 1,900 | 2,100 | 2,300 |
| Annual production (MWh) | 5,280.0 | 6,080.0 | 6,720.0 | 7,360.0 |
| PV EPC self-cost | €1,469,723 | €1,522,223 | €1,710,173 | €1,736,423 |
| Lighthief markup | €320,000 | €320,000 | €320,000 | €320,000 |
| PV EPC client price | €1,789,723 (€559,288/MWp) | €1,842,223 (€575,695/MWp) | €2,030,173 (€634,429/MWp) | €2,056,423 (€642,632/MWp) |
| BESS EPC (10 MWh) | €1,250,000 | €1,250,000 | €1,250,000 | €1,250,000 |
| Total CAPEX | €5,078,908 | €5,133,508 | €5,328,976 | €5,356,276 |
| Gross Y1 revenue | €760,168 | €875,605 | €967,682 | €1,059,758 |
| Y1 OPEX | €123,795 | €124,068 | €125,045 | €125,181 |
| Y1 EBITDA | €560,356 | €663,976 | €745,869 | €828,601 |
| Y1 FCF (post-tax) | €523,976 | €614,972 | €687,803 | €760,358 |
| Cash yield Y1 | 10.3% | 12.0% | 12.9% | 14.2% |
| Simple payback | 9.7 yrs | 8.3 yrs | 7.7 yrs | 7.0 yrs |
| Permit amendment? | No | No | Yes | Yes |

# 4. Option B Sensitivity Scenarios

Option B is the recommended base configuration. Two variables control whether payback falls under 9 years:

| Scenario | Discharge | Acq. Cost | Total CAPEX | Y1 FCF | Cash Yield | Payback |
| --- | --- | --- | --- | --- | --- | --- |
| B1 — Baseline | €195/MWh | €550k/MW | €5,133,508 | €614,972 | 12.0% | 8.3 yrs |
| B2 — Higher discharge | €220/MWh | €550k/MW | €5,133,508 | €660,136 | 12.9% | 7.8 yrs |
| B3 — Lower acquisition | €195/MWh | €500k/MW | €4,967,108 | €614,367 | 12.4% | 8.1 yrs |
| B4 — COMBINED ★ | €220/MWh | €500k/MW | €4,967,108 | €659,531 | 13.3% | 7.5 yrs |

★ B4 COMBINED best-case: CAPEX €4,967,108  |  FCF €659,531/yr  |  Payback 7.5 years
€220/MWh is still very conservative — April 2026 DAM observed >€340/MWh. €500k/MW acquisition requires €160k concession from seller — negotiable as EPC package deal.

# 5. Recommendation

Option B — Fixed 25° + White Albedo
Delivers 1,900 kWh/kWp (+15% vs standard) for €52k additional CAPEX with no permit amendment. At baseline assumptions: 8.3-year payback, 12.0% cash yield. Combined scenario B4 achieves 7.5 years with defensible conservative pricing.

# 6. Key Pricing Sources

| Item | Source / Value |
| --- | --- |
| Panels | AE Solar 645W bifacial — €0.112/W EXW Rotterdam (user-quoted) |
| Inverters | Trikkis CY Pricelist Mar 2026 — SUN2000-330KTL-H1 pallet €7,329.01 (code 200354) |
| Fencing | Trikkis Mar 2026 — code 400127: €10/m wholesale. Gate 400128: €300 |
| DC cables | Trikkis Mar 2026 — 4mm²: €0.52/m, 6mm²: €0.76/m wholesale |
| Civil works | User range €90-110k — modelled at €100k midpoint |
| MV transformer | User range €80-120k — modelled at €100k midpoint |
| BESS EPC | Lighthief rate €125k/MWh × 10 MWh |
| RTB acquisition | Seller asking price €550k/MW (scenario B3/B4 tests €500k/MW) |
| EAC grid works | EAC preliminary estimate €83,842 ex VAT — NOT BINDING (changes after TES) |
| Discharge price | TSOC DAM Oct 2025–Feb 2026 avg €183/MWh. Modelled: €195 (base) / €220 (B2/B4) |
| CIT | 12.5% Cyprus statutory corporate income tax |

Generated 11 May 2026  |  Lighthief Cyprus Ltd (HE 477423)  |  CONFIDENTIAL — INTERNAL USE ONLY