|  | Ref: LH-CY-PVBESS-APR2026 April 2026 · Strictly Confidential |
| --- | --- |

INVESTMENT TEASER  ·  CYPRUS RES MARKET

Cyprus PV + Battery Energy Storage

4-Hour Hybrid Connection  ·  8-Hour Flexible Nighttime Dispatch  ·  AE Solar Meteor N-type TOPCon Bifacial  ·  ≤8 MW Standard EAC Tier

Confidential - prepared by Lighthief Cyprus Ltd for qualified investors. Indicative figures based on confirmed EPC pricing and TSOC market data; not a binding offer.

| DAM EVENING PEAK €183/MWh TSOC sample Oct–Feb 2026 | BESS ROUND-TRIP EFFICIENCY 86.32% AC-AC · Linyang LFP | PV SPECIFIC YIELD 2,150 kWh/kWp Bifacial TOPCon · 25° · albedo 0.70 | PARK SIZE CAP ≤ 8 MW Standard EAC connection tier |
| --- | --- | --- | --- |

| MODEL A · 4H HYBRID Hybrid PV + BESS Connection Standard hybrid grid licence. PV exports directly during daytime at DAM rates. BESS captures curtailed energy (~65% curtailment) and discharges at evening peak. Dual revenue: direct PV sales + BESS arbitrage. ☀ PV 35% → Grid + 🔋 65% curtailed → BESS → 🌙 €183/MWh night | MODEL B · 8H FLEXIBLE Flexible Nighttime-Only Dispatch CERA flexible connection: zero daytime grid export. 100% of PV generation flows into BESS. All energy discharged overnight at DAM peak. Operationally simpler; eliminates midday price exposure; larger BESS required (8h duration). ☀ 100% PV → BESS → 🌙 100% night dispatch → €183/MWh |
| --- | --- |

⚡ Why nighttime dispatch? TSOC DAM sample (134 trading days, Oct 2025–Feb 2026): 57 of 134 days recorded zero or negative midday prices. Evening peak averaged €182.99/MWh vs €101.13/MWh at midday — an 81% premium. BESS-backed nighttime dispatch eliminates midday price risk entirely.

MODEL A — 4-HOUR HYBRID CONNECTION PV day sales + BESS curtailment arbitrage · Linyang LFP 4h

| Metric | 1 MW + 4 MWh | 2.5 MW + 10 MWh | 5 MW + 20 MWh |
| --- | --- | --- | --- |
| CAPITAL EXPENDITURE |  |  |  |
| PV EPC — AE Solar Meteor TOPCon (€720/kWp) | €720,000 | €1,800,000 | €3,600,000 |
| BESS EPC — Linyang LFP 8h (V4 internal × 1.40 margin) | €600,000 (€150k/MWh) | €1,250,000 (€125k/MWh) | €2,400,000 (€120k/MWh) |
| RTB licence acquisition (€380k/MW) | €380,000 | €950,000 | €1,900,000 |
| EAC grid connection terms (€80k/MW) | €80,000 | €200,000 | €400,000 |
| MV cable to substation (1 km × €20k/km — site variable) | €20,000 | €20,000 | €20,000 |
| Permitting & documentation | €35,000 | €35,000 | €35,000 |
| Total CAPEX | €1,835,000 | €4,255,000 | €8,355,000 |
| ANNUAL REVENUE (YEAR 1) |  |  |  |
| PV production (AE Meteor · 2,150 kWh/kWp) | 2,150 MWh | 5,375 MWh | 10,750 MWh |
| BESS nighttime discharge (RTE 86.32% × 100% production) | 1,146 MWh | 2,865 MWh | 5,730 MWh |
| Gross revenue (DAM evening peak × €182.99/MWh) | €209,708 | €524,269 | €1,048,538 |
| Less: aggregator fee (10%) | −€31,572 | −€78,930 | −€157,860 |
| Net Annual Revenue | €284,148 | €710,370 | €1,420,739 |
| OPERATING COSTS & EBITDA |  |  |  |
| PV O&M (€15/kWp/yr · cleaning + inspection) | −€15,000 | −€37,500 | −€75,000 |
| BESS LTSA (€1,740/MWh/yr · 97% availability · 15yr) | −€6,960 | −€17,400 | −€34,800 |
| EBITDA | €262,188 | €655,470 | €1,310,939 |
| Return Metrics |  |  |  |
| Unlevered payback | 7.0 yr | 6.5 yr | 6.4 yr |
| Equity required (35% of CAPEX) | €642,250 | €1,489,250 | €2,924,250 |
| Senior debt (65% · 5.5% p.a. · 12yr annuity) | €1,192,750 | €2,765,750 | €5,430,750 |
| Annual debt service | −€138,394 | −€320,908 | −€630,126 |
| DSCR | 1.89× | 2.04× | 2.08× |
| Net cash to equity (post 15% CIT) | €118,987 | €316,290 | €641,354 |
| Equity payback (levered) | 5.4 yr | 4.7 yr | 4.6 yr |

MODEL B — 8-HOUR FLEXIBLE CONNECTION 100% nighttime dispatch · zero daytime export · Linyang LFP 8h

| Metric | 1 MW + 8 MWh | 2.5 MW + 20 MWh | 5 MW + 40 MWh |
| --- | --- | --- | --- |
| CAPITAL EXPENDITURE |  |  |  |
| PV EPC — AE Solar Meteor TOPCon (€720/kWp) | €720,000 | €1,800,000 | €3,600,000 |
| BESS EPC — Linyang LFP 8h (V4 internal × 1.40 margin) | €1,208,000 (€151k/MWh) | €2,820,000 (€141k/MWh) | €5,120,000 (€128k/MWh) |
| RTB licence acquisition (€380k/MW) | €380,000 | €950,000 | €1,900,000 |
| EAC grid connection terms (€80k/MW) | €80,000 | €200,000 | €400,000 |
| MV cable to substation (1 km × €20k/km — site variable) | €20,000 | €20,000 | €20,000 |
| Permitting & documentation | €35,000 | €35,000 | €35,000 |
| Total CAPEX | €2,443,000 | €5,825,000 | €11,075,000 |
| ANNUAL REVENUE (YEAR 1) |  |  |  |
| PV production (AE Meteor · 2,150 kWh/kWp) | 2,150 MWh | 5,375 MWh | 10,750 MWh |
| BESS nighttime discharge (RTE 86.32% × 100% production) | 1,856 MWh | 4,640 MWh | 9,279 MWh |
| Gross revenue (DAM evening peak × €182.99/MWh) | €339,607 | €849,019 | €1,698,037 |
| Less: aggregator fee (10%) | −€33,961 | −€84,902 | −€169,804 |
| Net Annual Revenue | €305,647 | €764,117 | €1,528,234 |
| OPERATING COSTS & EBITDA |  |  |  |
| PV O&M (€15/kWp/yr · cleaning + inspection) | −€15,000 | −€37,500 | −€75,000 |
| BESS LTSA (€1,740/MWh/yr · 97% availability · 15yr) | −€13,920 | −€34,800 | −€69,600 |
| EBITDA | €276,727 | €691,817 | €1,383,634 |
| Return Metrics |  |  |  |
| Unlevered payback | 8.8 yr | 8.4 yr | 8.0 yr |
| Equity required (35% of CAPEX) | €855,050 | €2,038,750 | €3,876,250 |
| Senior debt (65% · 5.5% p.a. · 12yr annuity) | €1,587,950 | €3,786,250 | €7,198,750 |
| Annual debt service | −€184,249 | −€439,316 | −€835,265 |
| DSCR | 1.50× | 1.57× | 1.66× |
| Net cash to equity (post 15% CIT) | €92,478 | €252,501 | €548,368 |
| Equity payback (levered) | 9.2 yr | 8.1 yr | 7.1 yr |

TECHNOLOGY & EPC CAPABILITY Lighthief — exclusive Linyang Energy distributor in Cyprus · 28 parks / 134 MW / 496.5 MWh pipeline

| SOLAR PV — AE SOLAR METEOR |  | BESS — LINYANG LFP |  |
| --- | --- | --- | --- |
| Module | AE Solar Meteor N-type TOPCon bifacial | Chemistry / cells | LFP (EVE) · EN 50549-2 TÜV cert. |
| Wattage / efficiency | 620–630 Wp · η 22.9% | PCS models | 1.0 / 1.25 MW C-series |
| Bifaciality / albedo | 80% · white ground 0.70 | RTE | 86.32% AC-AC (full system) |
| Specific yield | 2,150 kWh/kWp · 25° south | Cycle life | 7,000 cycles @ 70% EOL DoD |
| Degradation | 0.4%/yr (N-type — low) | SoH Yr5 / 10 / 15 | 85% / 79.6% / 70% guaranteed |
| Product warranty | 30 years | Base warranty | 5 years (in EPC price) |
| PV EPC | €720/kWp all-in turnkey | LTSA | €1,740/MWh/yr · 97% avail. · 15yr |
|  |  | Incoterms | CIF Limassol · 90d prod + 50d transit |
|  |  | Pipeline | 28 parks · 134 MW · 496.5 MWh confirmed |

INDICATIVE PROJECT TIMELINE From RTB acquisition to Provisional Acceptance Certificate (PAC)

| RTB / EPC Contract Month 0 | Linyang Production +90 days | CIF Limassol +50 days | Installation & Commissioning +60–90 days | PAC — Grid Connected ~Month 9 |
| --- | --- | --- | --- | --- |

MV cable: illustrative 1 km × €20,000/km — adjust for site distance. RTB + EAC connection terms apply for standard tier ≤8 MW. Above 8 MW, higher EAC connection cost tiers apply. All figures indicative; formal EPC proposal on request.

| Contact: Alexander Papacosta Phone: +357 99 164 158 Email: alexander.papacosta@lighthief.com Web: lighthief.cy | Lighthief Cyprus Ltd · HE 477423 28 October Ave 249, Lophitis Business Center, Office 201 3035 Limassol, Cyprus |
| --- | --- |